# INSTITUTE OF ECE ECA14 – EMBEDDED SYSTEMS

| Exp. No | Name of the Experiment | Remarks |
| --- | --- | --- |
| 1 | Flashing of Led using AT89C51 Microcontroller using Proteus |  |
| 2 | Generation of Square Wave using Proteus |  |
| 3 | Stepper Motor Using AT89C51 using Proteus |  |
| 4 | 7 Segment Display Using AT89C51 using Proteus |  |
| 5 | Arduino based Manual Electronic Counter using Proteus |  |
| 6 | Arduino to 16x2 LCD Display using Proteus |  |
| 7 | Serial Communication using Arduino with Proteus |  |
| 8 | Gas Sensor MQ-2 in Proteus using Arduino |  |
| 9 | Ultrasonic Sensor in Proteus using Arduino |  |
| 10 | PIR Sensor in Proteus using Arduino |  |
| 11 | Study of GPIO & Bit Addressing in AT89C51 using Embedded C |  |
| 12 | Embedded C program to multiply two 16-bit binary numbers. |  |
| 13 | Embedded C program to add an array of 16-bit numbers and store the 32-bit result in internal RAM. |  |
| 14 | Embedded C program to display “Hello World” message |  |
| 15 | Embedded C program to convert the hexadecimal data to decimal and display the digits on ports P0, P1 and P2 |  |
| 16 | Serial Interrupt Using UART (Keyboard Input) In Keil |  |
| 17 | UART Interfacing using Interrupt in KEIL |  |

List of Experiments

| 18 | Generation of PWM using TIMER Interrupt |  |
| --- | --- | --- |
| 19 | Blinking LEDs using software delay routine in LPC2148 kit |  |

| 20 | Program to read the switch and display in the LEDs using LPC2148 kit |  |
| --- | --- | --- |
| 21 | Display a number in seven segment LED in LPC2148 kit |  |
| 22 | Serial transmission and reception using on-chip UART in LPC2148 kit |  |
| 23 | Access an internal ADC and display the binary output in LEDs in LPC2148 kit |  |
| 24 | Modeling Clock-Driven Concurrent Processes Using SC_THREAD and wait() in SystemC |  |
| 25 | Producer–Consumer Problem Using SystemC |  |
| 26 | Transaction Level Modeling (TLM) using SystemC |  |
| 27 | Binary Semaphore in SystemC |  |
| 28 | Counting Semaphore in SystemC |  |
| 29 | Priority-Based Interrupt Handling |  |
| 30 | RTOS Task Scheduling |  |

Exp No: 1

# FLASHING OF LED USING AT89C51 MICROCONTROLLER USING PROTEUS

AIM:

To Write an assembly language program to flash the LED using AT89C51

# SOFTWARES REQUIRED:

- Proteus software

# PROGRAM

ORG 0000H UP: SETB P2.0

ACALL DELAY CLR P2.0 ACALL DELAY SJMP UP

DELAY: MOV R4,#35 H1:MOV R3,#255 H2:DJNZ R3,H2

DJNZ R4,

H1 RET

END

Exp No: 2

# AIM:

GENERATION OF SQUARE WAVE USING PROTEUS

To write an assembly language program to Generate square wave using AT89C51.

# PROGRAM

ORG 0000H UP: SETB P2.0

ACALL DELAY CLR P2.0 ACALL DELAY SJMP UP

DELAY: MOV R4,#35        H1:MOV R3,#255 H2:DJNZ R3,H2

DJNZ R4,

H1 RET

END

# Exp No: 3: STEPPER MOTOR USING AT89C51 USING PROTEUS

AIM:

To write an assembly language program for Stepper Motor Using AT89C51 using Keil and Proteus

# PROGRAM:

ORG 0000H

UP: MOV P2,#09H ACALL DELAY MOV P2,#0CH ACALL DELAY MOV P2,#06H ACALL DELAY MOV P2,#03H ACALL DELAY SJMP UP DELAY:MOV R4,#18 H1:MOV R3,#255 H2:DJNZ R3,H2 DJNZ R4,H1

RET END

Exp No: 4

# 7 SEGMENT DISPLAY USING AT89C51 USING PROTEUS

AIM:

To write an assembly language program for 7 Segment Display Using AT89C51 using Keil and Proteus

# PROGRAM:

ORG 000H UP:MOV P2,#0C0H ACALL DELAY MOV P2,#0F9H ACALL DELAY MOV P2,#0A4H ACALL DELAY MOV P2,#0B0H ACALL DELAY MOV P2,#99H ACALL DELAY MOV P2,#92H ACALL DELAY MOV P2,#82H ACALL DELAY MOV P2,#0F8H ACALL DELAY MOV P2, #80H ACALL DELAY MOV P2,#90H ACALL DELAY

DELAY: MOV R5,#10 H1:MOV R4,#180 H2:MOV R3,#255 H3:DJNZ R3,H3 DJNZ R4,H2

DJNZ R5,H1 RET

END

Exp No: 5

# ARDUINO BASED MANUAL ELECTRONIC COUNTER USING PROTEUS

# AIM:

To write an Embedded C program for Manual Electronic counter using Arduino IDE and Proteus

# PROGRAM:

int x0,x1,x2,x3,x4,x5,x6,x7,x8,x9; int delay_time=200;

void setup() {

//configure pin2 as an input and enable the internal pull-up resistor pinMode(12, INPUT_PULLUP);

pinMode(1,OUTPUT); pinMode(2,OUTPUT);

pinMode(3,OUTPUT); pinMode(4,OUTPUT); pinMode(5,OUTPUT); pinMode(6,OUTPUT); pinMode(7,OUTPUT);

}

void loop() {

//read the pushbutton value into a variable int sensorVal = digitalRead(12);

if (sensorVal == LOW) { x0=true;

}

while(x0){ zero();

sensorVal = digitalRead(12); if (sensorVal == LOW) {

x1=true; x0=false;

}

}

while(x1){ one();

sensorVal = digitalRead(12); if (sensorVal == LOW) {

x2=true; x1=false;

}

}

while(x2){ two();

sensorVal = digitalRead(12); if (sensorVal == LOW) { x3=true; x2=false;

}

}

while(x3){ three();

sensorVal = digitalRead(12); if (sensorVal == LOW) { x4=true; x3=false;

}

}

while(x4){ four();

sensorVal = digitalRead(12); if (sensorVal == LOW) { x5=true; x4=false;

}

}

while(x5){ five();

sensorVal = digitalRead(12); if (sensorVal == LOW) { x6=true; x5=false;

}

}

while(x6){ six();

sensorVal = digitalRead(12); if (sensorVal == LOW) { x7=true; x6=false;

}

}

while(x7){ seven();

sensorVal = digitalRead(12); if (sensorVal == LOW) { x8=true; x7=false;

}

}

while(x8){

eight();

sensorVal = digitalRead(12); if (sensorVal == LOW) { x9=true; x8=false;

}

}

while(x9){ nine();

sensorVal = digitalRead(12); if (sensorVal == LOW) {

x0=true; x9=false;

}

}

}

void zero()

{

for(int i=1;i<7;i++)

{

digitalWrite(i,HIGH); digitalWrite(7,LOW);

}

delay(delay_time);

}

void one()

{

digitalWrite(1,LOW); digitalWrite(2,HIGH);

digitalWrite(3,HIGH); digitalWrite(4,LOW); digitalWrite(5,LOW); digitalWrite(6,LOW); digitalWrite(7,LOW); delay(delay_time);

}

void two()

{

digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,LOW); digitalWrite(4,HIGH); digitalWrite(5,HIGH); digitalWrite(6,LOW);

digitalWrite(7,HIGH); delay(delay_time); }

void three()

{

digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,HIGH); digitalWrite(4,HIGH); digitalWrite(5,LOW); digitalWrite(6,LOW);

digitalWrite(7,HIGH); delay(delay_time); }

void four()

{

digitalWrite(1,LOW); digitalWrite(2,HIGH); digitalWrite(3,HIGH); digitalWrite(4,LOW); digitalWrite(5,LOW); digitalWrite(6,HIGH); digitalWrite(7,HIGH);

delay(delay_time);

}

void five()

{

digitalWrite(1,HIGH); digitalWrite(2,LOW); digitalWrite(3,HIGH); digitalWrite(4,HIGH); digitalWrite(5,LOW);

digitalWrite(6,HIGH); digitalWrite(7,HIGH); delay(delay_time); }

void six()

{

digitalWrite(1,HIGH); digitalWrite(2,LOW); digitalWrite(3,HIGH); digitalWrite(4,HIGH); digitalWrite(5,HIGH); digitalWrite(6,HIGH); digitalWrite(7,HIGH); delay(delay_time);

}

void seven()

{

digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,HIGH); digitalWrite(4,LOW); digitalWrite(5,LOW);

digitalWrite(6,LOW); digitalWrite(7,LOW); delay(delay_time); }

void eight()

{

digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,HIGH); digitalWrite(4,HIGH); digitalWrite(5,HIGH); digitalWrite(6,HIGH); digitalWrite(7,HIGH); delay(delay_time);

}

void nine()

{

digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,HIGH); digitalWrite(4,HIGH); digitalWrite(5,LOW);

digitalWrite(6,HIGH); digitalWrite(7,HIGH); delay(delay_time);

}

Exp No: 6

ARDUINO TO 16x2 LCD DISPLAY USING PROTEUS

# AIM:

To write an Embedded C program to interface Arduino Uno and 16 x 2 LCD display using Arduino Uno and Proteus

# PROGRAM:

Arduino to 16x2 LCD Display using Proteus #include <LiquidCrystal.h>

LiquidCrystal lcd (7, 8, 9, 10, 11, 12);

// your PWM numbers may be different void setup() {

lcd.begin(16, 2);

lcd.print("put your message here");

}

void loop () {

for (int i=0; 1<13; i++)

}

lcd.scrollDisplayLeft(); delay(700);

}

}

Exp No: 7

# SERIAL COMMUNICATION USING ARDUINO WITH PROTEUS

AIM:

To write an Embedded C program for serial communication using Arduino Uno and Proteus

# PROGRAM:

void setup()

{

Serial.begin(9600); // send and receive at 9600 baud

}

int number = 0; void loop()

{

Serial.print("Number is "); Serial.println(number); // print the number delay(500); // delay half second between numbers number++; // to the next number

}

Exp No: 8

# GAS SENSOR MQ-2 IN PROTEUS USING ARDUINO

AIM:

To write an Embedded C program for interfacing Gas Sensor MQ-2 using Arduino Uno and Proteus

# PROGRAM:

#define GreenLed 13 #define Sensor A0

void setup(){ pinMode(13,OUTPUT); pinMode(12,OUTPUT); pinMode(A0,INPUT);

Serial.begin(9600);

}

void loop() {

int value = analogRead(A0);

Serial.print("Analogic Value coming from the sensor : "); Serial.println(value);

delay(100); if(value>600){ digitalWrite(13,HIGH);

}

else{ digitalWrite(13,LOW);

}

delay(20);

}

Code for MQ6 sensor:

int mq6Pin = A0; // MQ-5 analog pin connected to A0

void setup() { Serial.begin(9600);

}

void loop() {

int sensorValue = analogRead(mq5Pin); Serial.print("MQ5 Sensor Value: "); Serial.println(sensorValue); delay(1000); // 1-second delay

}

Exp No: 9

# ULTRASONIC SENSOR IN PROTEUS USING ARDUINO

AIM:

To write an Embedded C program for interfacing Ultrasonic Sensor using Arduino Uno and Proteus

PROGRAM:

// Program Sketch int trig = 10;

int echo = 9; long duration; int cm;

void setup() {

// put your setup code here, to run once: pinMode(trig, OUTPUT);

pinMode(echo,INPUT);

Serial.begin(9600);

}

void loop() { // Sketch to calculate ultrasonic distance digitalWrite(trig, LOW);

delayMicroseconds(10); digitalWrite(trig, HIGH); delayMicroseconds(10); digitalWrite(trig, LOW); delayMicroseconds(10); duration = pulseIn(echo, HIGH); cm = (duration/2) * 0.034;

// Print Value on Serial Monitor Serial.print(" Distance = "); Serial.print(cm); Serial.println(" cm");

}

Exp No: 10

# PIR SENSOR IN PROTEUS USING ARDUINO

AIM:

To write an Embedded C program for interfacing PIR Sensor using Arduino Uno and Proteus

PROGRAM:

int Status = 13; // Digital pin D6

int sensor = 12; // Digital pin D7

void setup() {

Serial.begin(9600);

pinMode(sensor, INPUT); // declare sensor as input

pinMode(Status, OUTPUT); // declare LED as output

}

void loop() {

long state = digitalRead(sensor);

Serial.println(state) ;

if(state == HIGH) {

digitalWrite (Status, HIGH);

Serial.println("Motion detected!");

delay(1000);

}

else {

digitalWrite (Status, LOW);

Serial.println("Motion absent!");

delay(1000);

}

}